IE450                                      						             Dr. R. A. Wysk

Manufacturing Systems              				    			Homework #1

Engineering Design and Process Specification              	     Due:  January 19, 2006

Purpose:  The purpose of this assignment is to demonstrate your understanding of process engineering and engineering design specification.  Process engineering is an important prerequisite for Lean Manufacturing, i.e., it is difficult to understand how to improve a process if you do not understand why the product is being made in the manner in which it is produced.

Problem:  For the part shown in #1, create the process plan(s) if the part of the next page were to be produced in quantities of 100 on the Haas machining centers in the PSU FAME lab.  Make sure to calculate (or estimate) all of the time components required to make the part.

Table 1.  Process accuracy for the Haas VF series machines

| PROCESS | DIMENSIONAL ACCURACY + | POSITIONAL ACCURACY |  |
| --- | --- | --- | --- |
| DRILLING | 0.008 | 0.008 |  |
| REAMING | 0.003 | (AS PREVIOUS) |  |
| SEMI-FINISH BORING | 0.005 | 0.005 |  |
| FINISH BORING | 0.0005 | 0.0005 |  |
| COUNTER-BORING (SPOT-FACING) | 0.005 | 0.005 |  |
| END MILLING | 0.005 | 0.007 |  |

B

LARGE HOLE

- D -

2.000  + 0.010

1.0 BSC							3 HOLES EQUALLY

SPACED

2.0 BSC				       A

1.500 DIA

4.000 + 0.010

0.25 + 0.010

C

# PART INFORMATION

## PART NUMBER	LARGE HOLE			3 HOLES	QUANTITY

|  | # DIA | TRUE POSITION MMC | # DIA | # TRUE POSITION MMC | # QTY |
| --- | --- | --- | --- | --- | --- |
| 1 | 1.000 + 0.005 | 0.010 A-B-C | 0.250 + 0.003 | 0.010 A-B-C | 100 |
| 2 | 1.000 + 0.003 | 0.005 A-B-C | 0.250 + 0.005 | 0.005 D | 100 |