Nicholas Kutchi

Duke Univ. Med School

Mike Barber

Clearly define points

A lot of existing images

Prelim renderings

Look at where it might exist somewhere else to sell/reinforce ideas

Lobby

- make stronger visual statement – section studies of pendant volume vs. space volume

- what about lighting the beams on the ceiling – 3D effects to continue beam/boxing of beams idea

Auditorium

- cove in concept design

- controls?

- Kill direct light, just have cove glow – another level of ambient illumination instead of pendant fixture

Lounge/Café

- show design reduces trespass out

- don’t downlight an arch b/c of curvature of the architecture

Sandra

- Idea of gothic not played out in lobby

- Liked lobby fixture glow but the walls are glowing which will be neat but architects don’t like holes in beams

- No exterior lighting on the tower?  Light gothic tower – it’s a huge exterior feature – light it up – key to the building and “gothic” architecture

- What about orientation of spaces in the building and going from one to another?

- Overall images – could have been stronger from tower – more in rendering, auditorium needs more light on seating – clear on what you’re going for and how you solved it

Auditorium

- controls, look at whole space graphically

- seats look really dark – main surface to be lit is not lit – how are you holding the whole space together?

Andrea

- Lobby fixture could have huge potential with its presence in the space – gives lots of options for different sources and flexibility

- Define where light is going then look at pendant

- Problem with beams with holes b/c it is no longer a structural-looking beam

- Images too small – can’t see them

- Text size inconsistent

Auditorium

- speaker wall lit but what about speaker modeling?

- Think about how many ceiling planes you’re creating – too many?  Consider pendant plane at the same plane of the cove

Café

- says auditorium on slide

- better image – looks like barn

- careful with arch lighting – detracts from multiple layers

- arches already stand out by being in silhouette

- consider small quantity of large pendants instead of no pendants at all

- not very gothic without pendants

Patio

- second conceptual sketch – excellent sketch!

- Follow through with second idea and scketch

- First concept not good to people on ground but good idea if you could see from above the trees