Derek DiPiazza

Construction Management

September 8, 2006

Senior Thesis

Building Stats I

________________________________________________________________________

Building Name:		Warrenton Aquatics and Recreation Facility

Location and Site:		Old Waterloo Road

Warrenton, Virginia

Fauquier County

Building Occupant Name:	Town of Warrenton

Function Type:		New Recreation Center/Indoor Swimming Pool

Size:				58,738 ft2

# Stories Above Grade:	3 Total-	Lower Level- 35,976 ft2 -1/2 below grade

Upper Level- 14,832 ft2

Penthouse level- 8,930 ft2

Primary Project Team:	Owner- Town of Warrenton Public Works

GC- Forrester Construction

Arch.-Grimm and Parker Architects- John McBryde Hill

Civil Engineer- Rickmond Engineering Inc.

Structural Engineer- Elliot, Leboeuf + Associates

Mech/Elect- B2E Consulting Engineers

Pool Design- Counsilman/Hunsaker + Associates

Recreation Consultants- The Sports Management Group

Cost Estimating- Downey + Scott Associates, LLC

Dates of Construction:	January 10, 2006 to July 7, 2007

General Cost:			$22 million

Project Delivery Method:	Design Bid Build

ARCHITECTURE

Architecture (Des. & Func.):The new building is designed to provide a recreation facility for the town of Warrenton. It will consist of a competition swimming pool with spectator seating, leisure pool complete with waterslide and lounge areas, juice bars, fitness rooms, and office space to run the facility. It also features a cylindrical entrance vestibule on the upper floor as its main entrance which enhances this aesthetically pleasing building.

Major National Codes:	Virginia uniform statewide building code“USBC” 2000ed.

NFPA 101 Life Safety Code 2000

Zoning:			B, A-3, A-4   Un-separated, Mixed-Use

Historical Requirements:	N/A

Building Envelope:

Wall Systems Description:	The walls consist of CMU block backing with stone veneer base, brick veneer, metal shingles, aluminum framed glazed windows, cast stone, and architectural steel.

Roofing Systems Desc.-	Roof I- Natatorium Areas

Standing Seam Metal Roof (SSM)

Self Adhering Underlayment

3” Board Insulation

Vapor Barrier

½” Glass Reinforced Gypsum Substrate Board

Metal Deck (3”)

Roof II- Natatorium Areas, Light Monitor

Standing Seam Metal Roof (SSM)

Self Adhering Underlayment

3” Board Insulation

Vapor Barrier

½” Glass Reinforced Gypsum Substrate Board

Metal Deck (6”)

Roof III- Upper Level & Penthouse Areas

Standing Seam Metal Roof (SSM)

Self Adhering Underlayment

3” Board Insulation

Metal Deck (3”)

Roof IV- Upper Level Flat Roof Areas

Multiple Ply Built-up Roof System

Tapered Insulation (Slope)

3” Board Insulation

3” Metal Deck and 5 ½” Composite Deck

Roof V- Plaza Area (Green Roof)

Landscaping Material

Fabric Mat Root Barrier

Drainage and Water Retention Mat

Fabric Mat Root Barrier

3” Board Insulation (R-19)

Roofing Membrane

5 ½” Composite Deck