IE450                                                                                                Spring 2006

Assignment #3			        	                               Dr. R. A. Wysk

Due: February 2, 2006			Reference:  Slide Set#2 - metrics

1.  In a production cell dedicated to the production of a product family the following data corresponding to 10 working days has been obtained (the working time is 24 hours/day). The theoretical production was 14,500 units/day.

| Day | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Breakdowns (h) | - | 2 | 5 | - | - |
| Setup time (h) | 2 | 1 | - | 2 | 2 |
| Defects (units) | 100 | 150 | 200 | 100 | 150 |
| Total production (units) | 12,000 | 13,280 | 11,300 | 12,100 | 14,000 |

| Day | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- |
| Breakdowns (h) | - | 2 | - | 3 | - |
| Setup time (h) | - | 2 | 3 | 4 | - |
| Defects (units) | 220 | 130 | 80 | 140 | 200 |
| Total production (units) | 13,000 | 12,300 | 11,900 | 13,100 | 11,700 |

After a large technical investment, a new theoretical production rate of 20,000 units/day was fixed. After another 10 working days the following data was collected.

| Day | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| Breakdowns (h) | - | 3 | - | - | 4 |
| Setup time (h) | - | 3 | - | 2 | - |
| Defects (units) | 200 | 250 | 100 | 130 | 230 |
| Total production (units) | 15,000 | 13,200 | 15,600 | 14,800 | 15,200 |

| Day | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- |
| Breakdowns (h) | - | 4 | - | 1 | - |
| Setup time (h) | 2 | 3 | 2 | 2 | 2 |
| Defects (units) | 125 | 240 | 320 | 100 | 180 |
| Total production (units) | 14,800 | 12,000 | 15,600 | 15,100 | 14,900 |

- How has this change varied the Overall Equipment Efficiency (OEE) rate?

- Has this change been worthwhile the technical investment?

## 2.  PRECISION CORP. is a company dedicated to parts manufacturing. Its facility layout is shown in the following figure. In the presented tables some important data is also shown.

The company managers want to carry out a “methods and time study” to reduce the idle time. As a previous step, the managers need to analyze the flow process chart of the most important parts.

| Sections |  |  |  | Bill of operations of part P7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Code | Number of machines | Machine Type |  | Part | Operation (sequential order) | Self-control | Section code |
| 1 | 2 | Milling |  | P7 | 1 | NO | 2 |
| 2 | 1 | Lathe |  | P7 | 2 | SI | 1 |
| 3 | 2 | Grinding |  | P7 | 3 | SI | 2 |
| 4 | - | Quality |  | P7 | 4 | NO | 3 |
|  |  |  |  | P7 | 5 | - | 4 |

- Indicate what layout type the company has. Justify the answer and indicate the advantages and disadvantages of this layout type.

- Draw the flow process chart of the P7 part.